Date: 4-24-90  3:35pm

From: Glenn Connery:SPD:3Com

To: RELADMIN

cc: martina pavloff, glennc

Subj: IBMTOK 2.0(c) submittal

------------------------------------------------------------------

Below is the submittal form for v 2.0(c) of the IBM Token Ring MAC

driver.  This update is for the Microsoft 2.0 Shrink-Wrap and NDDK

releases, CS-008.  It fixes a bug in the DOS driver (see below).

------------------------------------------------------------------

DATE: April 24th, 1990

PRODUCT: IBM Token Ring MAC Driver (OS/2 and DOS)

TARGETED FOR DEFINED RELEASE:

ECO -

Release - CS-008

Microsoft 2.0 Shrink-Wrap

Microsoft 2.0 NDDK

Update Disk -

FILENAME:           PREVIOUS VERSION:        NEW VERSION:

=========           =================        ============

IBMTOK.OS2              2.0(b)                  2.0(c)

IBMTOK.DOS              2.0(b)                  2.0(c)

LIST OF FILES THAT CHANGED:

INIT.ASM, EQU.MAC

EXECUTABLE LOCATION: (sharename,pswrd,directory)

sharename is \\iditerod\ibmtok

directory is \2.0c

OS/2 driver is os2\product\ibmtok.os2

DOS driver is dos\product\ibmtok.dos

SOURCE LOCATION: (sharename,pswrd,directory)

sharename is \\iditerod\ibmtok

directory is \2.0c

DATE AND TIME OF EXECUTABLE/FILES:

OS/2 driver:

IBMTOK   OS2    17108   4-24-90   2:59p

DOS driver:

IBMTOK   DOS     9232   4-24-90   2:58p

CODE AND DATA MEMORY REQUIREMENTS:

Varies based on parameters.  Using the default parameter values

approximately 8K.  However, this can increase up to 15K if

large values are configured for MaxTransmits.  This would normally

only be used on a heavily loaded server.

DEPENDENCIES AND BUILD INSTRUCTIONS:

- Tool requirements:

MAKE    X045

MASM    5.10

LINK    5.01.20 or 5.01.21

MAPSYM  4.00

EXE2COM a replacement for EXE2BIN which is compatible with

any version of DOS in use (use 1.04 or later)

- Libraries:

DOSCALLS.LIB    from MASM 5.10 or MSC 5.10 or equivalent

in INC subdirectory

- Include files:

DEVHLP.INC      from OS/2 BAK or DDK

DOSMAC.INC      from OS/2 BAK or DDK

DEVSYM.INC      from OS/2 BAK or DDK

in INC subdirectory

- Build Instructions:

1.  Setup T: to a TOOLS sharename (eg. \\SIRIUS\TOOLS)

2.  Setup G: to a GLOBALS sharename (eg. \\PORSCHE\OS2GLOB)

3.  Execute MKOS2.BAT or MKDOS.BAT, optionally supplying

"DEV" (uppercase) as an argument if you want to build

the development version instead of PRODUCT.

Note that if you had last built a different version (DEV instead

of PRODUCT, or DOS instead of OS/2, etc) you will have to do

something to cause all of the files to be rebuilt before using

the batch files (e.g. "TOUCH *.ASM" or "DEL *.OBJ").

DESCRIPTION OF CHANGES:

Change to DOS driver to correct a problem with the code which

relocates the INIT code out of the way of volatile data structures

being allocated.  Since the copy operation overlapped the code

was copied incorrectly, resulting in a crash after load.

REFERENCE BUG NUMBER:

N/A

CHANGES IMPLEMENTED BY:

Glenn Connery

KNOWN BUGS (This includes parts of the file or program that have

not been completed):