# Designing the USB stacks for

# Microsoft Invisible Computing

### Abstract

These are some thoughts on how we can support USB controller chips in the system.  It relies on incomplete information therefore it is very preliminary.  The basic ideas are to leverage the namespace as much as possible, to reduce the number of components (COBs) in use while still maximizing code sharing, and to minimize the amount of code needed to support a new USB Host Controller Interface or a new USB Device Controller.

### 1. The USB tree as a NameSpace

The Universal Serial Bus is a tree of connected entities, with HUBs as nodes and Devices as leafs. The USB root is a Host Controller Interface (HCI) with at least one root HUB.

Figure 1 depicts our hypothetical working example.

EBI bus

KEYBD

MOUSE

P0

HUB0

HUB1

P0

P1

HUB2

P0

P1

P1

Figure 1

P2

P3

On the Atmel EB63’s EBI bus we have two HCI chips, each with a two-ported root HUB.  These are HUB0 and HUB1 in the picture.  A mouse “MOUSE” and an external four-ported HUB “HUB2” are connected to HUB0.  A keyboard “KEYBD” is connected to HUB2.  Nothing is connected to HUB1.

A USB device, including a HUB, is identified by three numbers: the Vendor ID, the Product ID and the Device ID.  In our example, we will assume these Ids are as follows:

$MOUSE is “VEN_054e_PID_0409_DEV_0105” – a Microsoft USB mouse

$KEYBD is “VEN_054e_PID_kkkk_DEV_kkkk” – a Microsoft USB keyboard

$HUB0 is “VEN_cccc_PID_cccc_DEV_cccc” – The Cypress SL811 HCI

$HUB1 is “VEN_8086_PID_iiii_DEV_iiii” – an Intel HCI

$HUB2 is “VEN_xxxx_PID_xxxx_DEV_xxxx” – some unspecified other HUB.

We shall use the pseudo variables above as shorthand in the corresponding strings, as they are fairly long.

We can map these USB trees in our namespace as follows:

“\usb\port0_”$HUB0”\port0_”$MOUSE

“\usb\port0_”$HUB0”\port1_”$HUB2”\port0_”$KEYBD

“\usb\port1_”$HUB1

Every HUB is a namespace, an object used to keep track of what is connected to it.

The root “usb” namespace is described later on, but you might think of it as a virtual root of roots, which is a HUB again.  The actual USB devices are the leafs and they cannot be queried for the INameSpace interface.  Note that one of the terminals of our example tree is a HUB, not a device.  In other words, our tree can have nodes with no leafs attached.  As mentioned, a HUB is a device therefore it can be queried for more than just the INameSpace interface.

### 2. Procedures and COBs

Let us consider the following four procedures the system will have to be able to perform: Autoconfig, Plug-in, MethodCall and Removal.

Autoconfig

The first case is what happens when the system boots, or when we want to start things up by hand (using SR0.EXE).  The goal here is to recognize and handle the HCI chips.

Recall what happens when the auto configuration code sees the following two lines, in the order given:

“sl”, “SL811.COB”, 0x20000000, IRQ30, ???

“hub”, “USBHUB.COB”, 0, STACKED, “sl”, 0

First we start the driver for the Cypress HCI, passing the addresses and interrupt numbers found on the EB63 board. This is registered as “sl” in the namespace.

Then we start the USB HUB controller driver, a common component that has no HCI hardware dependencies and is therefore shared by all HCIs.  We pass as argument the name of the underlying HCI and the (virtual) port number where it is connected.  This second component is registered as “hub” in the namespace.

This is the same HUB1 component that is started on top of the Intel HCI driver, something like this

“uhci”, “UHCI.COB”, 0xnnnnnn, IRQn, ???

“hub1”, “USBHUB.COB”, 0, STACKED, “uhci”, 0

Hidden point: the first time it is started the USBHUB.COB will create the top level “\usb” namespace.  NOTE: This is not currently done.

“SL”

SL811.COB

Port0

“MS”

USBMOUSE.COB

“HUB”                                   USBHUB.COB

Port0

Figure 2

The top portion of Figure 2 shows the relationship between the “HUB” component and the “SL” component, namely that the first knows about the second but not vice versa.

The third COB of Figure 2 is described in the next section.

Plug-in

The second case is what happens when we insert $MOUSE as indicated in Figure 1.

In this case, as well as in the case of a removal, it is the HCI hardware that is notified, most likely with a hardware interrupt.  The sequence of events, in a cryptic summary is as follows:

- INSERT interrupt to SL  wakeup the HUB thread passing (INSERT, PORT0)

- HUB  SL::GET_DESCRIPTOR  (VEN_054e,…)

- Lookup(VEN_054e,..)  “USBMOUSE.COB”

- Register( [“USBMOUSE.COB”, PORT0, ?args?] as “port0_VEN_054e…”

The hardware interrupt is recognized as being an insertion event, which happened on port number 0.  The HUB component on startup creates a thread to handle all events that require potential blocking and that cannot therefore be handled in an ISR.  The SL component does not create threads of its own, so this thread performs double-duty.

The tread wakes up, recognizes the event and returns from the GetEvent method of the SL driver.

Once back in HUB code, the thread turns around and starts interrogating the newly attached device using as many USB bus transactions as is necessary.  This provides the Ids of the device as well as the power requirements (mA of electrical current).  We assume we have enough power to support this device. NB: GET_PORT_STATUS here!

The next step is to identify which driver will handle the device.  Since this is not a USB HUB but a real device we must lookup the Ids in some plug-and-play database and find that the USBMOUSE.COM component is the appropriate one.  While the Lookup function is relatively simple, it will be desirable to make it flexible and extendable.

Once the component is identified, it is started and the resulting IUnknown object is registered in the namespace at the correct point in the tree.  This is the HUB component itself, as it is true in general.

MethodCall

The third case is what happens when someone wants to actually use the USB device.  For simplicity, we assume we would poll the mouse asking for any movement. In reality things operate more asynchronously but that is not a concern for us now.

- Mouse::ReadReport(ptr)  MS

- MS  HUB(PORT0, GET_REPORT, ptr)

- HUB  SL(PORT0,GET_REPORT,ptr)

- wait for USB bus transaction

- complete calls in reverse order

One way to handle things is to follow the hardware topology in software and route calls “in reverse”.  Starting at the MS mouse component we invoke the HUB component on its port0 attachment point. Eventually we reach the SL component on its port0 and this starts the hardware level routing process.

There are some drawbacks to this approach.  Aside from the unnecessary deep level of call forwarding we also have multiple points where a “REMOVE” event might catch this call in progress.  It might be desirable therefore to short-cut this routing and connect directly the MS component to the SL component. NOTE: This is not done yet.

Another consideration is that it might be desirable to pack the arguments to the various calls in an NT-like USB Request Block (URB, twin of IRPs).  Notice that it is almost by definition an unknown what are all the possible USB transactions.  This is both because the standard evolves and because new hardware devices and new USB transactions are created all the time.  This makes this very much like the IOCTL problem of old.

Removal

The fourth case is what happens when we remove the $MOUSE device.

- REMOVE interrupt to SL  wakeup HUB thread passing (REMOVE, PORT0)

- HUB Lookup(PORT0)  MS

- HUB  MS::RemoveYourself()

This is very similar to the INSERT event, except the Lookup() function is trivially the one that internally knows which device, if any, is attached to a given port number. Once that is identified we ask it to remove itself from the system.

Note that HUBs can be removed too, therefore they must have a RemoveYouself() method as well. In other words, they are USB device drivers just as well.

### 2. Usability

The horrible strings described above are useful for analyzing the state of things (with the tzk’s DIR command, say) but they do not solve a more obvious problem: how to get hold of the mouse quickly, from an application, ignoring any topological and religious considerations.

We propose to put some intelligence inside the endpoint drivers themselves and let them create symbolic links in the namespace.  In our example we might get

“\mouse”   “\usb\port0_”$HUB0”\port0_”$MOUSE

“\keyboard”  “\usb\port0_”$HUB0”\port1_”$HUB2”\port0_”$KEYBD

Recall that a “symbolic link” in our system is simply the same object registered twice.

Notice also that an important point here is how to remove those links when a device is unplugged.  This must be done in the RemoveYourself() method and before anything else is done to avoid race conditions.

An alternate idea would be to create/remove these links in the HUB drivers themselves, making them recognize somehow the “class” of a given device.  This requires much more and more questionable code though.  The USB committee has all but abandoned their initial idea of classifying all possible devices with a triple (class,subclass,protocol), which was the initial thought on how to build “generic” software drivers.

“SL”

SL811.COB

Port0

“MS”

USBMOUSE.COB

“HUB”                                   USBHUB.COB

Port0                      Port1

Figure 3

“HUB2”                                   USBHUB.COB

Port0

“UHCI”                                   UHCI.COB

Port0

“KBD”

USBKEYBD.COB

“HUB1”                                   USBHUB.COB

“mouse”

MOUSE.COB

“keyboard”

KEYBOARD.COB

The Human Interface Device (HID) specs in the USB stack are huge and it is next to impossible to quickly tell apart a mouse from a joystick from a wheel.  The HID group uses a “language” to describe the Device Reports, which is the way in which say a mouse reports a coordinate change or a button press, or both at once. This language is similar to XML in purpose, but radically different in practice. Most importantly, it is not possible to define additional “verbs” which would carry semantic information in an open-ended way.  The few existing semantic specifiers are crummy and limited, for instance it is only possible to have one “X” axis on a device, which causes trouble on the two-joysticks Xbox gamepad.

A better idea would be to specify “mouse” in the plug-and-play table that maps the (VEN,PID,DEV) triple to a COB.  But what then of a multifunction device, such as a mouse with a scrolling wheel?  Or the Xbox gamepad, how do we know which portion of the Report should be considered the “mouse” one.

### 3. Packing into COBs

Figure 3 depicts all of the components that would be instantiated in our example.

Each HCI has its own specific driver, as do each of the two USB devices.  There are three instances of the USBHUB component.  The devices of this example have existing generic ones to top them and they are shown bolded in the picture.  Note that this is not always required,  a USB device can be used directly by application code.  It is often desirable to split generic code and package it separately, as this leads to a potentially smaller footprint.

There are some packaging considerations to be made, as is often the case .

One might consider packaging together SL811 and USBHUB into a single COB, to be loaded at once and/or possibly be compiled directly into the base image.  This has two drawbacks: (a) if we had a second HCI to support we would get USBHUB code duplications and (b) if we attached a HUB to the system again we would get USBHUB code duplications.

It seems more appealing to package USBMOUSE and MOUSE, under the assumption that we only ever use a single mouse in a system, if at all.  Similarly for the USBKEYBD and KEYBOARD components.

### 4. The Source Tree View

The source tree is (to be) organized in two sub-trees, one for the host (master) side and one for the peripheral (slave) side.  The USB protocol definition is the only header file, others are generated from the XML descriptions of the various interfaces (see later).

We provide generic implementations of the host-side drivers for some common peripherals, like mouse and keyboard, as well as the specific drivers for some Host Controllers.  Currently we only have the Cypress SL811 example and to make things worse on the TAMU63 board it can only handle low-speed USB1.x devices because of the imprecision associated with using a 12 MHz crystal.

On the slave side, we have a driver for the Intel UDC (Universal Device Controller, sic) that is found on the CERF board (a.k.a. iPaq ). We have tested code to simulate a serial line, a mouse and a keyboard.

The tree looks as follows:

include\drivers\usb.h				;; USB1/2 Protocol definitions

src\drivers\serial\usb\slave\usbslave.xml	;; interface definitions

src\drivers\serial\usb\master\usbmaster.xml

src\drivers\serial\usb\master\sl811		;; Cypress host controller driver

src\drivers\serial\usb\master\<others like uhci,ohci,ehci>

src\drivers\serial\usb\master\usbhub		;; HUB driver

src\drivers\serial\usb\master\usbmouse	;; mouse driver

src\drivers\serial\usb\master\usbkeybd	;; keyboard driver

src\drivers\serial\usb\slave\udc		;; Intel’s slave controller driver

src\drivers\serial\usb\slave\<others like ezusbd,cy7c63xxx >

src\drivers\serial\usb\slave\usbdmouse	;; slave mouse driver example

src\drivers\serial\usb\slave\usbdserial		;; slave serial line driver

src\drivers\serial\usb\slave\usbdkeybd	;; slave keyboard example

### 5. The Slave Side

The old monolithic UDC.COB was split into multiple COBs, the device-specific code living in separate peripheral drivers.  Consider the case of the serial line emulation:

“com1”

USBDSERIAL.COB

“udc”                                   UDC.COB

Figure 4

IDevice

IFile

IUsbSlaveController

IDevice

IFile

IUsbSlaveDriver

As Figure 4 illustrates, for any specific case we will have only two COBs instantiated; the base UDC driver and one of the peripheral emulations.  Notice that the peripheral emulations are generic, they will work with any device controller chip.  On the slave side there is no need for hierarchies since in the USB world it does not make any sense to simulate multiple peripherals at once.  The case of a HUB is also a single peripheral case, at least as far as the point of attachment to the host side (slave) is concerned.

The UDC exports the IDevice interface for the benefits of the auto configuration code.  It also does implement the ReadAt/WriteAt methods of the IFile interface, for the benefit of the peripheral (serial) driver.  The main interface is the IUsbSlaveController, which will enforce a single-client rule.

The peripheral driver exports the IUsbSlaveDriver interface to the UDC and the regular IFile interface to its clients.  The interfaces are as follows:

IUsbSlaveController {

TakeOwnership(IUsbSlaveDriver Owner);

ReleaseOwnership(IUsbSlaveDriver Owner);

QueryEndpoints(out {int NumCtl, NumIn, NumOut} );

QueryEndpointProperties(Type,Index, out {int MaxPacket, IsSynch…});

}

IUsbSlaveDriver {

GetDescriptor( in Type {STRING,DEVICE,REPORT,ENDPOINT},

int Index,

out BYTE **Desc, out Size);

SetDescriptor( in Type {…}, int Index, in BYTE *Desc, in Size);

HandleRequest( in USB_REQUEST *Req,

in out BYTE **Data,

in out UINT *Size,

out Enum {COMPLETED, REFUSED, NEEDS_INPUT} );

}

The UDC starts first and waits for a peripheral to shows up.

The peripheral queries for as much info as necessary to build its descriptors then it takes ownership of the controller and it is ready to work.

The controller, when the cable is connected will ask the peripkeral for various descriptors. They are returned by reference and sent off to the host.  The host will also possibly ask to change them, though this funny idea is not used in practice.

The UDC handles all standard requests but must delegate to the peripheral (via HandleRequest) all class-level requests and requests that it does not know how to handle.

The HandleRequest method can return one of three answers.  If the request is simple (no further data needed) the peripheral handles it and returns COMPLETED.  If the request is unknown even at the class level, it is REFUSED.  If the request needs a data phase (such as SET_REPORT for instance) the peripheral returns NEEDS_INPUT and a buffer (pointer+size) where to return the data.  The UDC now completes the data phase and later comes back with the same request, this time with a non-null pointer to the buffer containing the data.  The peripheral can now repeat the same dance as above, COMPLETED or REFUSED or ask for even more data.

Sending data back to the host is handled in a similar, symmetrical way.

### 6. The Host Side

Figure 5 illustrates the COBs and the interfaces in (a subset of) our running example.

“usb0”

USBHUB.COB

“sl811”                                   SL811.COB

Figure 5

IDevice

IFile

IUsbMasterController

IDevice

IFile

IUsbHub

IUsbDriver

INameSpace

“mouse0”

USBMOUSE.COB

IDevice

IFile

IUsbDriver

The interfaces are as follows:

IUsbMasterController {

SubmitUrb( PUSBM_REQUEST_BLOCK Urb, int PortNumber);

CancelUrb( PUSBM_REQUEST_BLOCK Urb, int PortNumber);

}

IUsbHub :: IUsbMasterController {

QueryPorts( int *nPorts);

}

IUsbDriver {

IamYourHub ( IusbHub Hub, int PortNumber);

YouAreTerminated ( IusbHub SaysWho);

GetUsbAddress(int *pUsbAddress);

}

NOTE on Hubs and INameSpace

As indicated, every USB HUB must implement INameSpace to show its attached devices.  We implemented a “delegated” namespace solution, leveraging the default namespace to save code. Every HUB driver that is started first creates a regular NameSpace entry for itself with Bind() and it saves it as this->NameSpace. The HUB then Register()s this (itself) on top of it, using the same name used in the Bind() operation. The QueryInterface method will return this->NameSpace when asked for the INameSpace interface.

This way the device itself is in the visible namespace, but INameSpace is trivially delegated to this->NameSpace.  The attached devices are Register()ed in this->NameSpace.

Doing this has some advantages over purely delegating to the default namespace.

Basically, a directory node can be QI for various things, like say IFile and "type"d.

So you could "type \usb0" and get "Cypress sl811 root hub". This could also be accomplished by registering "this" under "hub.txt".  So you'd "type \usb0\hub.txt" instead and get the same result.

A problem with this idea is how actual devices are "type"d. Meaning, all USB devices can do GET_STRING_DESC, which is useful at least for debugging purposes. How is that exposed?  The two ideas above cover HUBs only. INameSpace would let us control that, say for each <device> we could show two entries: "<device>" and "<device>.info"

But we would have to implement the IFiles for the ".info" things.

Bottom line: its a lot of permanent code.  Would be better to write a program that gets the UsbAddress and uses UsbHub::SubmitUrb to find the descriptive strings. This program would be rarely used but would say a lot of things in addition to what we could do via builtin tricks.  This is indeed what we did, the USBDIR.EXE tool does just that.

Note that neither of these two alternate ideas can solve the bootstrap problem, which is how to get from the device's PartialName ("<dev>") to the FullName, which is something like "\usb0\hub1\hub1\...\<dev>".  Using the delegated INameSpace this is no longer necessary, because evey hub basically keeps a “pointer” to its location in the namespace tree in the form of the suindicated this->NameSpace.

### 6. Plug-and-play Manager

Dynamic driver startup requires some run-time lookup tables to associate a physical device with the component (driver) that will handle it.  This is currently known by the catch phrase "plug and play", to underline the desired simplicity of use.

Various busses use different ways to identify their devices, but it general comes down to two or three small integers that identify the manufacturer and device type and perhaps revision. In more standardized cases it might be a class and protocol identifiers pair.  Whatever the semantic of those small integers, it does not really concern us; all we need to provide is an association between that triple and the name of a component.

One idea for this is to use a namespace, using a string-ified form of the triplet for lookup.  However it is the implemented inside, the user will just do something like this:

sprintf(DriverName,"/Pnp/Vid%04x_Pid%04x_Dev%04x",

VendorID,ProductID,DeviceID);

BindToObject(CurrentNameSpace(),DriverName,&IID_IFile,&File);

File->ReadAt(File,..,&CobName);

In other words, the triple is used to lookup an object in the namespace (which could be anything). At this time we are just asking this object to export a way to return the corresponding device driver name, which we can easily do with an IFile.

Perhaps in the future this could become a different object, but for today that will do, and it has advantages.

Partial matching can be done with asterisks:

"/Pnp/Vid0452_Pid0666_*"

which would work for USB where there are usually only two IDs.

But getting too fancy is an overkill, I would NOT use something like this:

BindToObject( , "/Pnp/Vid0452_*", &IID_INameSpace, &Ns);

Dir(Ns) -> all driver names for Microsoft devices

especially since we can simply impose lexical ordering of strings, and use FindNext as customary.

Two simple advantages of this scheme is that a "dir \PnP" will show the list of all IDs the manager knows about, a simple practical problem that gets nonetheless complicated in Windows. And to find which driver handles a certain mouse you'd do just a "type \PnP\Vid045e_Pid0009_Dev*" and get "COB\usbmouse.cob".

Another advantage is that the table can be easily extended at runtime:

Register(CurrentNameSpace(),"\PnP\VenNNNN_PidMMMM_DevKKKK",File);

where "File" only has to implement ReadAt.

In that vein, we might have subspaces under \PnP. Say some future bus "SuperBus" uses not integers but some other form of identification. It could be implemented under say "\PnP\SuperBus\<superids>"

Another idea could be to directly start up the driver, without even knowing its name or how it is implemented. For instance, instead of QI for IFile one might QI for something else like say IDriverConstructor.  I mean, whatever you QI for we could start the component and then QI *it* for that IID.

But thats for tomorrow, today we keep it simple.

### 7. Of USB2 and HUBs

A few issues came about when adapting the host stack for the USB2.0 EHCI host controller.

- The maximum control packet size is not fixed at eight for USB2 and many devices exploit this.  This means that the device driver needs to know this information before it starts talking to its device, or perform some work to re-discover it.  The discovery process is one of asking for only 8 bytes of the device descriptor, looking at byte 7 and using that value in subsequent operations.  We avoided many code duplications this by passing the MCPS as an argument to all USB drivers, it was already available in the HUB driver that starts them because it needs more then the first 8 bytes of that descriptor.

- The EHCI needs to know the speed of an endpoint, whether it is the USB2 high-speed (480Mbps) or one of the USB1 full-speed (12Mbps) or low-speed (1.5Mbps).  This seems simple enough, but that information is only readily available at one point in the driver hierarchy: at the HUB driver where the device is attached.  At the same time, we wanted to make drivers oblivious to this issue so that they do not have to know their own speed before being able to compose their URBs.

The solution is a bit tricky, but it is handled entirely within the HUB driver.

- The HUB checks the device descriptor when a new device is found and notes if it is a HUB or not.

- The speed flags (low/full/high) are added in by a HUB driver in the SubmitUrb() method as follows.

- If the driver submitting the URB is not a HUB then the flags are added in.  This makes regular drivers oblivious to the speed issues.

- If the driver is a HUB and it is talking to itself (e.g. the destination UsbAddress is the same as the HUB’s UsbAddress) then the flags are also added in.  This is consistent with the above oblivion rule.  It also solves the problem for a root HUB that would have to ask the HCI driver below for its own speed.

- In all other cases, the flags are not affected.  This covers the case where the HUB driver directly above is the attach point for the device as well as the case where it is two or more levels above the device that is the target of the URB.

- The EHCI also needs to know the routing information for full-speed and low-speed devices.  The hardware data structures require both the normal destination Address/Endpoint pair and an additional pair indicating the HubAddress and the PortNumber on that HUB where the request should be routed.  The same hardware structures also require precise speed information, which is what created the problem described above.

Notice that the HUB in question is the one where we make the speed-domain shift, not the one where the device is attached.  This makes sense if we consider that USB is essentially a broadcast bus, and when the HCI speaks, everyone listens.  Both in USB1 and in USB2 there are provisions for isolating lower-speed devices from traffic at higher speeds.

Notice also that there is no such thing as a low-speed HUB; they only come in full-speed (USB1) and high-speed (USB2) types.

It is also the case that speed-domain transitions are only possible from higher to lower speeds, not the other way around.  If we attach a USB2 to a USB1 HUB, the USB2 HUB will either behave as a USB1 HUB for all practical purposes or cease to function.

Given all the above considerations the solution to the routing problem was to add the suindicated two additional fields to the URB and modify the SubmitUrb() method of the usbhub.cob device driver as follows.

1- SubmitUrb() looks at the speed of the device that is submitting the URB.

2- If the device is a high-speed device then the URB routing is not affected.  It is either left blank because the HCI will not need it, or it is unchanged from the last time it was modified.

3- In all other cases the routing information in the URB is modified as follows.

4- The HubDeviceAddress is set to the UsbAddress of the current HUB (e.g. this->MyUsbAddress).

5- The HubPortNumber field is set to the destination port number than is an argument to the SubmitUrb() method.

Figure 6 shows an example hierarchy of USB devices of differing speeds that covers all the cases of special interest for the speed and routing issues mentioned above.

An EHCI host controller “Ehci0” is managed by the ehci.cob device driver. The chip in this example has two ports, even though real EHCIs have usually more than two ports. The usbhub.cob driver is used as the root hub “Rh0” for the EHCI chip.

All ports in the figure are indicated by small circles and numbered according to the USB rules, starting at one for the lowest index port. Next to the port there is an indication of the port speed, which is either Low, Full or High.  Notice that the speed is known only to the left side of the vertical bar that depicts a HUB’s port set.  Hub1 in the Figure does not know that it is a high-speed device, only Rh0 knows that fact.

Various USB drivers are indicated by a shorthand of their name and a suffix, which is the USB address, assigned to them.  “Hub1” is a USB2 HUB and “Hub4” is a USB1 HUB, they are both managed by usbhub.cob.  “Mou2” and “Mou6” are both low-speed mice and managed by usbmouse.cob.  “Stor5” is a USB1 storage device (an USBkey for instance) and “Stor7” an analogous USB2 device. They are both managed by usbstor.cob.  “Cam3” is a USB2 camera, for which we do not actually have a device driver at the time of this writing.

Figure 6

Ehci0

Rh0

H

H

1

2

Hub1

Stor7

H

H

1

2

Mou65

Stor5

L

F

1

2

Hub4

Cam3

Mou2

F

L

H

1

2

3

A simple transaction is as follows.  Cam3 wishes to communicate with its endpoint number 3.  It fills in an URB leaving the speed flags empty, it addresses it to Device=3, Endpoint=3, Hub=0, HubPort=0 and calls SubmitUrb(Port=2) on its Hub1.  We can succinctly indicate this URB as [3,3,0,0](-,2).  According to the rules above, Hub1 notices that the port is a high-speed one and leaves the routing information unchanged. It also notices that Cam3 is not a HUB and therefore changes the speed flags before forwarding the URB on to Rh0. The URB has now become [3,3,0,0](H,1).

Rh0 again leaves the routing information unchanged and notices that Hub1 is a HUB but that it is not speaking to itself.  Therefore, it does not change the speed flags.  The URB is submitted to Ehci0 as [3,3,0,0](H,1).

Mou2 creates an URB for its endpoint 1 as [2,1,0,0](-,1) and submits it.  Hub1 changes it to [2,1,1,1](L,1), noticing that the port is not high-speed and therefore the routing information must be added in.  Similarly, the speed flags are set. At the Rh0 level the URB now becomes [2,1,1,1](L,1), same port only by coincidence.

Stor4 is a simple variation of the above sequence, if it wishes to communicate to its control endpoint it builds [5,0,0,0](-,1).  Hub4 changes it to [5,0,4,1](F,3), then Hub1 changes it to [5,0,1,3](F,1) and this is what Ehci0 sees.

A more subtle case is when a new device, say Mou6 is attached to a HUB.  The reserved pair UsbAddress=0, Endpoint=0 is initially used to configure the device and to assign it a valid UsbAddress but… is this the same endpoint used to configure Stor7 later?  The answer is no, the EHCI chip must create two distinct endpoint (QueueHead descriptors to be exact) to handle the two cases. The former is a low-speed endpoint, the latter a high-speed one. They differ both in the routing information and in the flags: [0,0,1,3](L,1) versus [0,0,0,0](H,2).

Other transactions are of interest, they are left to the reader as an exercise.

### 7. Example: the UsbNull driver

To be filled in later.

Describe structure, startup and shutdown.  Add material from mouse/keyboard/storage as appropriate.